The ABC Sweep

A + B → C in a pore space: the complete campaign specification

CompLaB3D v1.3.2 reactive transport, abiotic, second order, counter-current. Sixty runs over ten pore spaces, feeding a PRT-DeepONet-3D operator network.

Every number in this document was taken from the campaign as it was actually built: the geometries from geometries/index.csv, the run matrix from campaign/runs.csv, and the timesteps from the thirty calibration runs the solver itself performed on Sapelo2. Nothing here is a design target that was not met.

# 1. At a glance

Table 1. The campaign in one table.

| Quantity | Value | Note |
| --- | --- | --- |
| Reaction | A + B → C | R = k[A][B], irreversible, one step |
| Rate law | second order | abiotic; no microbes, no minerals, no equilibrium |
| Runs | 60 | 10 geometries × 3 Peclet × 2 Damkohler |
| Snapshots written | 21 | at iterations 0, V, 2V, ... 20V |
| Usable samples | 1200 | the frame at t = 0 is all zero and is dropped |
| Grid | 32 × 32 × 32 | 32768 voxels per field |
| Voxel size dx | 10 µm | domain L = 320 µm |
| Diffusivity D | 1 × 10⁻⁹ m²/s | the same for A, B and C, deliberately |
| Flow relaxation τ | 0.8 | ν = (τ − ½)/3 |
| Feed A0 = B0 | 1 mol/L | chosen so float16 holds five decades of the field |
| Boundaries | mixed | each species held at its feed face, open at its exit face |
| Timestep | measured, not assumed | 30 calibration runs, 0.00857 to 0.00898 s |
| Wall clock | about 6 core-hours | Slurm array, 20 tasks at a time |
| Output | about 4.6 GB | 77 MB per run of .vti, rate fields and checkpoints |

# 2. The two dimensionless numbers

Pe = U L / D and Daᵣ = kᵣ L² / D, both built on the same L, which is what makes them comparable. Damköhler needs a first-order constant, so the second-order k is collapsed against the feed: kᵣ = R(A0,A0)/A0 = k·A0, giving Da = k A0 L² / D and therefore k = Da·D/(A0 L²).

Table 2. The Peclet values, what each one does, and how long its runs take.

| Peclet | Meeting surface, x/L | B's reach from outlet | Physical end time | Steps |
| --- | --- | --- | --- | --- |
| 0.1 | ≈ 0.53 | the whole domain | 116.4 s | 12961 to 13581 |
| 0.5 | ≈ 0.66 | roughly half | 85.33 s | 9501 to 9961 |
| 2.0 | ≈ 0.88 | about six voxels | 42.67 s | 4761 to 4981 |

The meeting surface is where A and B cross when solved as conservative tracers. The sweep stops at Pe = 2 because CompLaB's Peclet is superficial: at porosity 0.36 a nominal 2 is an interstitial 5.6, and anything higher would collapse the reaction onto the outlet plane where the pore structure has nothing left to say about it.

Table 3. The Damkohler values and the rate constants they correspond to.

| Damkohler | k, L/(mol s) | What the field looks like | Fraction of feed consumed per step |
| --- | --- | --- | --- |
| 0.1 | 9.766 × 10⁻⁴ | slow: the plumes interpenetrate, C is weak and spread | ≈ 0.001 % |
| 10 | 9.766 × 10⁻² | fast: A and B annihilate on contact, C is a sheet | ≈ 0.09 % |

The per-step cap in the rate law trips at 25 % of the scarcer reactant, so even the fast case runs at three thousandths of it. The smoke run confirmed this: the positivity clamp limited 25 increments out of 18,513,228, which is 0.00 %.

# 3. The ten pore spaces

One family throughout: packs of overlapping spheres of radius 3 voxels, at five target porosities with two seeds each. One family rather than a mixture, because with ten geometries a mixture gives one or two of each, and one example of a structure teaches a network that the structure is a label rather than a variable.

Table 4. The ten sweep geometries, as generated. Porosity is over the duct interior; the block figure includes the confining wall.

| gid | Name | Target φ | Achieved φ | Block φ | Grains | Seed | Tortuosity | Split |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | geom_0000 | 0.36 | 0.3615 | 0.3177 | 411 | 1036 | 1.255 | train |
| 1 | geom_0001 | 0.36 | 0.3569 | 0.3137 | 400 | 1037 | 1.349 | train |
| 2 | geom_0002 | 0.42 | 0.4217 | 0.3706 | 352 | 1042 | 1.298 | VALIDATION |
| 3 | geom_0003 | 0.42 | 0.4198 | 0.3690 | 374 | 1043 | 1.267 | VALIDATION |
| 4 | geom_0004 | 0.48 | 0.4801 | 0.4220 | 303 | 1048 | 1.226 | train |
| 5 | geom_0005 | 0.48 | 0.4805 | 0.4223 | 288 | 1049 | 1.243 | train |
| 6 | geom_0006 | 0.54 | 0.5384 | 0.4732 | 266 | 1054 | 1.176 | TEST |
| 7 | geom_0007 | 0.54 | 0.5367 | 0.4717 | 246 | 1055 | 1.215 | TEST |
| 8 | geom_0008 | 0.60 | 0.5998 | 0.5271 | 192 | 1060 | 1.177 | train |
| 9 | geom_0009 | 0.60 | 0.5996 | 0.5270 | 210 | 1061 | 1.160 | train |

Every achieved porosity is within 0.004 of its target. Tortuosity falls monotonically with porosity, from 1.349 at φ = 0.357 to 1.160 at φ = 0.600, which is the geometric signal the network is being asked to read.

Table 5. The analytic reference geometry, run in its own campaign and not part of the dataset.

| gid | Name | Kind | Interior φ | Block φ | Tortuosity | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| 900 | geom_0900 | straight duct | 1.0000 | 0.8789 | 1.000 | closed-form check |

Built with every face held (--boundaries dirichlet), which is the only configuration whose answer can be written down in advance.

# 4. The split, held out by porosity

Table 6. How the sixty runs divide. Whole geometries are held out, never individual runs.

| Split | Porosity | Geometries | Runs | Samples | Run ids |
| --- | --- | --- | --- | --- | --- |
| train | 0.36, 0.48, 0.60 | 6 | 36 | 720 | 0000–0011, 0024–0035, 0048–0059 |
| validation | 0.42 | 2 | 12 | 240 | 0012–0023 |
| test | 0.54 | 2 | 12 | 240 | 0036–0047 |
| total | — | 10 | 60 | 1200 | 0000–0059 |

The split is drawn from porosity rather than at random. Two packings at the same porosity are close enough that splitting between them would leak the pore structure into the test set, and the score would measure recognition rather than transfer. Holding out 0.42 and 0.54, both interior to the training range, asks for interpolation.

Run ids are not arbitrary. Every geometry gets the same six parameter pairs in the same order, so the id tells you the case:

run id = 6 × gid + 2 × (Peclet index) + (Damkohler index)

# 5. Every run

The complete matrix, exactly as campaign/runs.csv holds it. The timestep in each row was measured by the solver in that geometry, not assumed; the step count follows from it and from the physical end time the design calls for.

Table 7. All sixty runs: geometry, parameters, measured timestep, step count and snapshot spacing.

| Run | gid | φ | Split | Pe | Da | k, L/(mol s) | dt, s | Steps | Every | End, s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 0 | 0.3615 | train | 0.1 | 0.1 | 0.0009766 | 0.0089727 | 12961 | 648 | 116.30 |
| 0001 | 0 | 0.3615 | train | 0.1 | 10.0 | 0.09766 | 0.0089727 | 12961 | 648 | 116.30 |
| 0002 | 0 | 0.3615 | train | 0.5 | 0.1 | 0.0009766 | 0.0089688 | 9521 | 476 | 85.39 |
| 0003 | 0 | 0.3615 | train | 0.5 | 10.0 | 0.09766 | 0.0089688 | 9521 | 476 | 85.39 |
| 0004 | 0 | 0.3615 | train | 2.0 | 0.1 | 0.0009766 | 0.0089491 | 4761 | 238 | 42.61 |
| 0005 | 0 | 0.3615 | train | 2.0 | 10.0 | 0.09766 | 0.0089491 | 4761 | 238 | 42.61 |
| 0006 | 1 | 0.3569 | train | 0.1 | 0.1 | 0.0009766 | 0.0088513 | 13141 | 657 | 116.31 |
| 0007 | 1 | 0.3569 | train | 0.1 | 10.0 | 0.09766 | 0.0088513 | 13141 | 657 | 116.31 |
| 0008 | 1 | 0.3569 | train | 0.5 | 0.1 | 0.0009766 | 0.0088495 | 9641 | 482 | 85.32 |
| 0009 | 1 | 0.3569 | train | 0.5 | 10.0 | 0.09766 | 0.0088495 | 9641 | 482 | 85.32 |
| 0010 | 1 | 0.3569 | train | 2.0 | 0.1 | 0.0009766 | 0.0088358 | 4821 | 241 | 42.60 |
| 0011 | 1 | 0.3569 | train | 2.0 | 10.0 | 0.09766 | 0.0088358 | 4821 | 241 | 42.60 |
| 0012 | 2 | 0.4217 | VALIDATION | 0.1 | 0.1 | 0.0009766 | 0.0088104 | 13201 | 660 | 116.31 |
| 0013 | 2 | 0.4217 | VALIDATION | 0.1 | 10.0 | 0.09766 | 0.0088104 | 13201 | 660 | 116.31 |
| 0014 | 2 | 0.4217 | VALIDATION | 0.5 | 0.1 | 0.0009766 | 0.0088175 | 9681 | 484 | 85.36 |
| 0015 | 2 | 0.4217 | VALIDATION | 0.5 | 10.0 | 0.09766 | 0.0088175 | 9681 | 484 | 85.36 |
| 0016 | 2 | 0.4217 | VALIDATION | 2.0 | 0.1 | 0.0009766 | 0.0088405 | 4821 | 241 | 42.62 |
| 0017 | 2 | 0.4217 | VALIDATION | 2.0 | 10.0 | 0.09766 | 0.0088405 | 4821 | 241 | 42.62 |
| 0018 | 3 | 0.4198 | VALIDATION | 0.1 | 0.1 | 0.0009766 | 0.0087834 | 13241 | 662 | 116.30 |
| 0019 | 3 | 0.4198 | VALIDATION | 0.1 | 10.0 | 0.09766 | 0.0087834 | 13241 | 662 | 116.30 |
| 0020 | 3 | 0.4198 | VALIDATION | 0.5 | 0.1 | 0.0009766 | 0.0087843 | 9721 | 486 | 85.39 |
| 0021 | 3 | 0.4198 | VALIDATION | 0.5 | 10.0 | 0.09766 | 0.0087843 | 9721 | 486 | 85.39 |
| 0022 | 3 | 0.4198 | VALIDATION | 2.0 | 0.1 | 0.0009766 | 0.0087821 | 4861 | 243 | 42.69 |
| 0023 | 3 | 0.4198 | VALIDATION | 2.0 | 10.0 | 0.09766 | 0.0087821 | 4861 | 243 | 42.69 |
| 0024 | 4 | 0.4801 | train | 0.1 | 0.1 | 0.0009766 | 0.0088297 | 13181 | 659 | 116.38 |
| 0025 | 4 | 0.4801 | train | 0.1 | 10.0 | 0.09766 | 0.0088297 | 13181 | 659 | 116.38 |
| 0026 | 4 | 0.4801 | train | 0.5 | 0.1 | 0.0009766 | 0.0088276 | 9661 | 483 | 85.28 |
| 0027 | 4 | 0.4801 | train | 0.5 | 10.0 | 0.09766 | 0.0088276 | 9661 | 483 | 85.28 |
| 0028 | 4 | 0.4801 | train | 2.0 | 0.1 | 0.0009766 | 0.0088131 | 4841 | 242 | 42.66 |
| 0029 | 4 | 0.4801 | train | 2.0 | 10.0 | 0.09766 | 0.0088131 | 4841 | 242 | 42.66 |
| 0030 | 5 | 0.4805 | train | 0.1 | 0.1 | 0.0009766 | 0.0089805 | 12961 | 648 | 116.40 |
| 0031 | 5 | 0.4805 | train | 0.1 | 10.0 | 0.09766 | 0.0089805 | 12961 | 648 | 116.40 |
| 0032 | 5 | 0.4805 | train | 0.5 | 0.1 | 0.0009766 | 0.0089752 | 9501 | 475 | 85.27 |
| 0033 | 5 | 0.4805 | train | 0.5 | 10.0 | 0.09766 | 0.0089752 | 9501 | 475 | 85.27 |
| 0034 | 5 | 0.4805 | train | 2.0 | 0.1 | 0.0009766 | 0.0089507 | 4761 | 238 | 42.61 |
| 0035 | 5 | 0.4805 | train | 2.0 | 10.0 | 0.09766 | 0.0089507 | 4761 | 238 | 42.61 |
| 0036 | 6 | 0.5384 | TEST | 0.1 | 0.1 | 0.0009766 | 0.0088330 | 13181 | 659 | 116.43 |
| 0037 | 6 | 0.5384 | TEST | 0.1 | 10.0 | 0.09766 | 0.0088330 | 13181 | 659 | 116.43 |
| 0038 | 6 | 0.5384 | TEST | 0.5 | 0.1 | 0.0009766 | 0.0088308 | 9661 | 483 | 85.31 |
| 0039 | 6 | 0.5384 | TEST | 0.5 | 10.0 | 0.09766 | 0.0088308 | 9661 | 483 | 85.31 |
| 0040 | 6 | 0.5384 | TEST | 2.0 | 0.1 | 0.0009766 | 0.0088199 | 4841 | 242 | 42.70 |
| 0041 | 6 | 0.5384 | TEST | 2.0 | 10.0 | 0.09766 | 0.0088199 | 4841 | 242 | 42.70 |
| 0042 | 7 | 0.5367 | TEST | 0.1 | 0.1 | 0.0009766 | 0.0087212 | 13341 | 667 | 116.35 |
| 0043 | 7 | 0.5367 | TEST | 0.1 | 10.0 | 0.09766 | 0.0087212 | 13341 | 667 | 116.35 |
| 0044 | 7 | 0.5367 | TEST | 0.5 | 0.1 | 0.0009766 | 0.0087187 | 9781 | 489 | 85.28 |
| 0045 | 7 | 0.5367 | TEST | 0.5 | 10.0 | 0.09766 | 0.0087187 | 9781 | 489 | 85.28 |
| 0046 | 7 | 0.5367 | TEST | 2.0 | 0.1 | 0.0009766 | 0.0087047 | 4901 | 245 | 42.66 |
| 0047 | 7 | 0.5367 | TEST | 2.0 | 10.0 | 0.09766 | 0.0087047 | 4901 | 245 | 42.66 |
| 0048 | 8 | 0.5998 | train | 0.1 | 0.1 | 0.0009766 | 0.0087374 | 13321 | 666 | 116.39 |
| 0049 | 8 | 0.5998 | train | 0.1 | 10.0 | 0.09766 | 0.0087374 | 13321 | 666 | 116.39 |
| 0050 | 8 | 0.5998 | train | 0.5 | 0.1 | 0.0009766 | 0.0087361 | 9761 | 488 | 85.27 |
| 0051 | 8 | 0.5998 | train | 0.5 | 10.0 | 0.09766 | 0.0087361 | 9761 | 488 | 85.27 |
| 0052 | 8 | 0.5998 | train | 2.0 | 0.1 | 0.0009766 | 0.0087268 | 4881 | 244 | 42.60 |
| 0053 | 8 | 0.5998 | train | 2.0 | 10.0 | 0.09766 | 0.0087268 | 4881 | 244 | 42.60 |
| 0054 | 9 | 0.5996 | train | 0.1 | 0.1 | 0.0009766 | 0.0085731 | 13581 | 679 | 116.43 |
| 0055 | 9 | 0.5996 | train | 0.1 | 10.0 | 0.09766 | 0.0085731 | 13581 | 679 | 116.43 |
| 0056 | 9 | 0.5996 | train | 0.5 | 0.1 | 0.0009766 | 0.0085742 | 9961 | 498 | 85.41 |
| 0057 | 9 | 0.5996 | train | 0.5 | 10.0 | 0.09766 | 0.0085742 | 9961 | 498 | 85.41 |
| 0058 | 9 | 0.5996 | train | 2.0 | 0.1 | 0.0009766 | 0.0085746 | 4981 | 249 | 42.71 |
| 0059 | 9 | 0.5996 | train | 2.0 | 10.0 | 0.09766 | 0.0085746 | 4981 | 249 | 42.71 |

“Every” is save_VTK_interval, the number of steps between snapshots. ade_max_iT = Every × 20 + 1 in every row, so the last write lands on the last step and all sixty runs hold the same 21 frames. Note that the step count varies with geometry while the end time does not: that is the point of measuring the timestep, and it is what makes t_norm mean the same moment in every sample.

# 6. The measured timesteps

CompLaB does not let you set the timestep. It computes refNu = PoreMeanU · L / Pe from the flow field, then ade_dt = refNu · dx² / D, so dt exists only after the flow has been solved in that particular pore space. These thirty numbers are what the solver printed when asked to run one step in each geometry at each Peclet.

Table 8. The timestep the solver computed, per geometry and Peclet, in seconds.

| gid | φ | Pe = 0.1 | Pe = 0.5 | Pe = 2.0 | Against the nominal 0.01 s |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.3615 | 0.0089727 | 0.0089688 | 0.0089491 | 0.896 × nominal, spread 0.26 % |
| 1 | 0.3569 | 0.0088513 | 0.0088495 | 0.0088358 | 0.885 × nominal, spread 0.18 % |
| 2 | 0.4217 | 0.0088104 | 0.0088175 | 0.0088405 | 0.882 × nominal, spread 0.34 % |
| 3 | 0.4198 | 0.0087834 | 0.0087843 | 0.0087821 | 0.878 × nominal, spread 0.03 % |
| 4 | 0.4801 | 0.0088297 | 0.0088276 | 0.0088131 | 0.882 × nominal, spread 0.19 % |
| 5 | 0.4805 | 0.0089805 | 0.0089752 | 0.0089507 | 0.897 × nominal, spread 0.33 % |
| 6 | 0.5384 | 0.0088330 | 0.0088308 | 0.0088199 | 0.883 × nominal, spread 0.15 % |
| 7 | 0.5367 | 0.0087212 | 0.0087187 | 0.0087047 | 0.871 × nominal, spread 0.19 % |
| 8 | 0.5998 | 0.0087374 | 0.0087361 | 0.0087268 | 0.873 × nominal, spread 0.12 % |
| 9 | 0.5996 | 0.0085731 | 0.0085742 | 0.0085746 | 0.857 × nominal, spread 0.02 % |

Within a geometry the three Peclet values agree to a fraction of a per cent, which is Stokes flow being linear. Between geometries they spread by about 4 per cent, and that spread is the part no estimate could have supplied. The nominal value, (τ − ½)/3 · dx²/D = 0.01 s, would have been 13 per cent too high.

# 7. The boundary conditions

Table 9. What each species does at each face. The sweep uses the mixed set.

| Face | A | B | C |
| --- | --- | --- | --- |
| x = 0, inlet | Dirichlet 1.0 (feed) | Neumann (outflow) | Neumann (outflow) |
| x = nx−1, outlet | Neumann (outflow) | Dirichlet 1.0 (feed) | Neumann (outflow) |
| y = 0 and ny−1 | wall, drawn into the geometry | same | same |
| z = 0 and nz−1 | wall, drawn into the geometry | same | same |
| Initial condition | 0.0 everywhere | 0.0 everywhere | 0.0 everywhere |

B is fed from the OUTLET face, so its Neumann is on the left. A Neumann on B's right would mean B never enters and nothing reacts; check_campaign.py refuses such a campaign. Nothing starts in the domain, which is what forces the counter-current arrangement in the first place: CompLaB offers Dirichlet boundaries on the two x-normal planes and nowhere else.

# 8. What each run leaves on disk

Table 10. Output per case, and what it is for.

| File | Count | Size each | What it holds |
| --- | --- | --- | --- |
| A_*.vti, B_*.vti, C_*.vti | 21 × 3 | ≈ 350 kB | concentration, mol/L, named after the species |
| rate_A_*.vti and friends | 21 × 3 | ≈ 350 kB | per-voxel reaction rate, mol/L/s |
| nsLattice_*.vti | 21 | ≈ 700 kB | velocity and velocityNorm |
| maskLattice_*.vti | 1 | ≈ 350 kB | the live pore mask at the end |
| *.chk | 5 | 2 to 5 MB | binary checkpoints, written unconditionally |
| summary.csv | 1 | small | one scalar row per snapshot |
| run.log | 1 | small | the whole solver log, including [KIN], Daᵣ and [CLAMP] |
| status.json | 1 | tiny | ok or failed, exit code, wall time |

About 77 MB per run and 4.6 GB across the campaign. The collector reads only the species and flow files; everything else is kept for inspection.

# 9. The dataset the collector builds

Table 11. dataset.h5, as PRT-DeepONet-3D reads it.

| Path | Shape | Type | What it is |
| --- | --- | --- | --- |
| /geom/gid | (10,) | int32 | which geometry each row is |
| /geom/material | (10, 32, 32, 32) | uint8 | 0 solid, 1 wall, 2 pore |
| /geom/gdf | (10, 32, 32, 32) | float32 | geodesic distance from the inlet |
| /geom/edt | (10, 32, 32, 32) | float32 | distance to the nearest solid |
| /samples/geom_index | (60,) | int32 | row into /geom |
| /samples/params | (60, 2) | float32 | Pe and Da |
| /samples/t_norm | (60, 20) | float32 | 0 to 1 of each run's own length |
| /samples/conc | (60, 20, 3, 32, 32, 32) | float16 | A, B and C, raw, in mol/L |
| /samples/velocity | (60, 3, 32, 32, 32) | float16 | the steady flow field |
| /samples/split | (60,) | string | train, val or test |
| conc_scale | (3,) | float32 | per-species max, an attribute on /samples |

Concentration is stored raw and conc_scale beside it, because dataset_reader.py divides by conc_scale when it hands a target to the model. A file that stored the already-divided field would be normalised twice.

# Provenance

Geometries: tools/make_geometries.py, all eleven round-tripped on shape, porosity, sealed y and z faces, no bare solid against pore, and x-slice ordering. Campaign: tools/make_campaign.py build with --calibration, thirty measured timesteps. Gate: tools/check_campaign.py check --complab, which confirmed the binary reads PRT_KABIO, that the Damkohler banner agrees with the campaign arithmetic, and that the flow solver hit its Peclet target. Submitted to Sapelo2 as Slurm array job 48553093.